# Sioux Falls (TNTP) → SUMO → MATLAB (TraCI4Matlab) Pipeline

Clean reproducible steps with commands and minimal scripts (Windows VM, SUMO 1.26.0, MATLAB 2025b)

# Overview

This document summarizes the working, end-to-end process to convert the TransportationNetworks Sioux Falls TNTP files into SUMO artifacts, generate routed vehicles, add traffic lights, and control/step the simulation from MATLAB using TraCI4Matlab.

# Inputs (TNTP)

Place these files in a single working folder (example shown below):

SiouxFalls_node.tntp

SiouxFalls_net.tntp

SiouxFalls_trips.tntp

Example working directory used in our setup:

\\psf\home\Documents\SUMO_Work\Predefined_Networks\TransportationNetworks-master\SiouxFalls

# One-time prerequisites

SUMO installed on Windows (SUMO 1.26.0).

Environment variable SUMO_HOME points to SUMO install root (contains bin\ and tools\).

TraCI4Matlab installed in C:\Matlab_Work\ (MATLAB +traci package + traci4matlab.jar).

Check SUMO version (PowerShell):

& "$env:SUMO_HOME\bin\sumo-gui.exe" --version

MATLAB: ensure jar + package are available (run each session):

javaaddpath("C:\Matlab_Work\traci4matlab.jar"); addpath(genpath("C:\Matlab_Work"));

# Step 1 — Import TNTP network to SUMO plain XML

Run the SUMO importer to create node/edge XML building blocks:

cd "\\psf\home\Documents\SUMO_Work\Predefined_Networks\TransportationNetworks-master\SiouxFalls" python "$env:SUMO_HOME\tools\import\transportationTestProblems.py" SiouxFalls_node.tntp SiouxFalls_net.tntp dir *.nod.xml, *.edg.xml

Expected outputs:

nodes.nod.xml

edges.edg.xml

# Step 2 — Build a valid SUMO network (.net.xml)

Create a valid SUMO network file from nodes+edges:

& "$env:SUMO_HOME\bin\netconvert.exe" --node-files ".\nodes.nod.xml" --edge-files ".\edges.edg.xml" -o ".\siouxfalls_clean.net.xml"

Sanity check: the file must contain <net version=...> (not netedit/netconvert configuration roots):

Select-String -Path .\siouxfalls_clean.net.xml -Pattern "^\s*<net\b" -List

# Step 3 — Fix geometry via UTM projection

The imported node coordinates are latitude/longitude degrees. For normal-looking geometry in SUMO, project to meters (UTM zone 14N).

Important note: in our environment, writing the projected net to the shared \\psf path was unreliable. Writing to C:\Temp was reliable.

& "$env:SUMO_HOME\bin\netconvert.exe" --node-files ".\nodes.nod.xml" --edge-files ".\edges.edg.xml" --proj "+proj=utm +zone=14 +datum=WGS84 +units=m +no_defs" -o "C:\Temp\siouxfalls_utm_clean.net.xml" Select-String -Path C:\Temp\siouxfalls_utm_clean.net.xml -Pattern "<location " ¦ Select-Object -First 1

# Step 4 — Create SUMO demand inputs: TAZ + tazRelation

SUMO tools require (a) a TAZ mapping file and (b) OD demand in SUMO’s tazRelation format.

Create a Python script make_sumo_od_inputs.py in the working folder and run it. It produces:

siouxfalls.taz.xml

siouxfalls.tazRel.xml

Python script: make_sumo_od_inputs.py

#!/usr/bin/env python3 import os, sys, re, argparse from pathlib import Path ORIGIN_RE = re.compile(r"^\s*Origin\s+(\d+)\s*$", re.IGNORECASE) PAIR_RE = re.compile(r"(\d+)\s*:\s*([0-9.eE+-]+)") ZONES = list(range(1, 25)) # 1..24 def require_sumo_tools(): sumo_home = os.environ.get("SUMO_HOME") if not sumo_home: raise RuntimeError("SUMO_HOME is not set.") sys.path.append(str(Path(sumo_home) / "tools")) import sumolib # noqa return sumo_home def read_net(net_path): import sumolib return sumolib.net.readNet(str(net_path)) def pick_edges_for_node(net, node_id: str): n = net.getNode(node_id) if n is None: return [], [] out_edges = [e.getID() for e in n.getOutgoing() if not e.getID().startswith(":")] in_edges = [e.getID() for e in n.getIncoming() if not e.getID().startswith(":")] return out_edges, in_edges def write_taz_xml(net, out_taz, weight=1.0): out_taz = Path(out_taz) lines = ['<?xml version="1.0" encoding="UTF-8"?>', "<tazs>"] for z in ZONES: zid = str(z) src, snk = pick_edges_for_node(net, zid) # assumes zone ids match node ids lines.append(f' <taz id="{zid}">') for e in src: lines.append(f' <tazSource id="{e}" weight="{weight}"/>') for e in snk: lines.append(f' <tazSink id="{e}" weight="{weight}"/>') lines.append(" </taz>") lines.append("</tazs>") out_taz.write_text("\n".join(lines) + "\n", encoding="utf-8") def parse_tntp_trips(tntp_path): pairs = [] cur_origin = None with open(tntp_path, "r", encoding="utf-8", errors="ignore") as f: for raw in f: line = raw.strip() if not line: continue m = ORIGIN_RE.match(line) if m: cur_origin = m.group(1) continue if cur_origin is None: continue for to_, val in PAIR_RE.findall(line): cnt = float(val) if cnt > 0: pairs.append((cur_origin, to_, cnt)) return pairs def write_tazrelation_xml(out_path, pairs, begin=0, end=3600, interval_id="car"): out_path = Path(out_path) lines = [ '<?xml version="1.0" encoding="UTF-8"?>', '<data xmlns:xsi="https://www.w3.org/2001/XMLSchema-instance" ' 'xsi:noNamespaceSchemaLocation="https://sumo.dlr.de/xsd/datamode_file.xsd">', f' <interval id="{interval_id}" begin="{begin}" end="{end}">' ] for fr, to, cnt in pairs: lines.append(f' <tazRelation from="{fr}" to="{to}" count="{cnt}"/>') lines += [" </interval>", "</data>"] out_path.write_text("\n".join(lines) + "\n", encoding="utf-8") def main(): require_sumo_tools() ap = argparse.ArgumentParser() ap.add_argument("--net", default="siouxfalls_clean.net.xml") ap.add_argument("--tntp", default="SiouxFalls_trips.tntp") ap.add_argument("--begin", type=float, default=0) ap.add_argument("--end", type=float, default=3600) args = ap.parse_args() net = read_net(Path(args.net)) write_taz_xml(net, "siouxfalls.taz.xml", weight=1.0) pairs = parse_tntp_trips(args.tntp) write_tazrelation_xml("siouxfalls.tazRel.xml", pairs, begin=args.begin, end=args.end, interval_id="car") print("Wrote siouxfalls.taz.xml and siouxfalls.tazRel.xml") if __name__ == "__main__": main()

Run the script (PowerShell):

python .\make_sumo_od_inputs.py --net .\siouxfalls_clean.net.xml --tntp .\SiouxFalls_trips.tntp --begin 0 --end 3600

# Step 5 — OD → trips (od2trips) and scaling for debugging

Generate trips from OD relations. Use --scale for smaller test runs (e.g., 0.1 = 10% demand).

od2trips -n .\siouxfalls.taz.xml -z .\siouxfalls.tazRel.xml -o .\siouxfalls_10p.trips.xml --begin 0 --end 3600 --spread.uniform --scale 0.1

# Step 6 — Define a vehicle type 'car' (for duarouter)

If trips reference type='car', duarouter needs a vType definition. Create vtypes.add.xml:

<?xml version="1.0" encoding="UTF-8"?> <additional> <vType id="car" vClass="passenger"/> </additional>

# Step 7 — Trips → routed vehicles (duarouter) on the projected network

Route trips on the same net you intend to simulate with. Use the UTM-projected net to get consistent geometry.

duarouter -n "C:\Temp\siouxfalls_utm_clean.net.xml" -r .\siouxfalls_10p.trips.xml -a .\vtypes.add.xml -o .\siouxfalls_10p_utm.rou.xml

# Step 8 — Add traffic lights with netconvert (avoid netedit config confusion)

Generate a valid TLS network using netconvert (instead of accidentally saving a netedit configuration).

& "$env:SUMO_HOME\bin\netconvert.exe" -s C:\Temp\siouxfalls_utm_clean.net.xml --tls.guess -o C:\Temp\siouxfalls_utm_tls_fixed.net.xml

Verify TLS exists in the net:

Select-String -Path C:\Temp\siouxfalls_utm_tls_fixed.net.xml -Pattern "<tlLogic\b" -List

# Step 9 — Create a SUMO config for GUI + MATLAB-driven stepping

Create siouxfalls_gui.sumocfg in the working folder (relative route path, absolute net path):

<?xml version="1.0" encoding="UTF-8"?> <configuration> <input> <net-file value="C:\Temp\siouxfalls_utm_tls_fixed.net.xml"/> <route-files value="siouxfalls_10p_utm.rou.xml"/> </input> <time> <begin value="0"/> <end value="3600"/> </time> </configuration>

# Step 10 — Start SUMO-GUI as a TraCI server (MATLAB drives steps)

Run SUMO-GUI and let MATLAB advance the simulation with traci.simulationStep().

cd "\\psf\home\Documents\SUMO_Work\Predefined_Networks\TransportationNetworks-master\SiouxFalls" & "$env:SUMO_HOME\bin\sumo-gui.exe" -c .\siouxfalls_gui.sumocfg --remote-port 8813 --num-clients 1 --delay 0

# Step 11 — MATLAB: connect, enumerate TLS, step the sim, and log metrics

Example MATLAB driver (connect + step + basic metrics).

% Reset state try, traci.close(); catch, end clear global connections message javaaddpath("C:\Matlab_Work\traci4matlab.jar"); addpath(genpath("C:\Matlab_Work")); % Connect to SUMO-GUI TraCI server [traciV, sumoV] = traci.init(8813, 10, '127.0.0.1', 'default'); disp(traciV); disp(sumoV); % TLS list (example found: {'10'}) tls_ids = traci.trafficlights.getIDList(); disp(tls_ids); nSteps = 5000; for k = 1:nSteps traci.simulationStep(); if mod(k,200)==0 t = traci.simulation.getTime(); dep = traci.simulation.getDepartedNumber(); arr = traci.simulation.getArrivedNumber(); active = numel(traci.vehicle.getIDList()); fprintf("t=%.1f departed=%d arrived=%d active=%d\n", t, dep, arr, active); end end traci.close(); clear global connections message

# Common pitfalls we avoided

Do not use netedit/netconvert configuration files as SUMO networks. A valid net must contain <net version=...>.

When opening in netedit, use: netedit -s <netfile.net.xml>. Using -n is for plain node files and yields 'No nodes loaded'.

Writing some outputs directly into the \\psf shared path was unreliable. Generating critical net outputs to C:\Temp worked reliably.

Avoid duplicate vType IDs: define 'car' either in the route file or via additional-files, but not both.

SUMO with --num-clients 1 may appear to 'pause' after starting the server until a client connects; this is normal.